# ISSCC2018 Invited Paper

# 13.1  A Shift Towards Edge Machine-Learning Processing

O. Temam, Google, Paris, France

(Presented Feb 13, 2018 at ISSCC 2018, San Francisco CA)

The field of machine learning, especially Deep Neural Networks, is advancing at a breathtaking pace, with new functionalities achieved on a monthly basis. In the span of a few years, close to human-level accuracy has been achieved for simple voice commands, then full speech recognition, speech synthesis, translation, and increasing progress has been achieved in language understanding.

Machine-learning researchers largely acknowledge that the current successes of deep neural networks have been fueled by two evolutions: the availability of a large quantity of data for training, and the availability of high-performance computing at low cost, initially enabled by GPUs. Both advances combined to make training times tractable for large neural-network models.

Beyond GPUs, both the broad application span and high computational cost of deep neural networks have made custom machine-learning hardware economically sensible, and such architectures are currently being developed by many hardware, cloud or startup companies. The number of competing companies and the broad dissemination of knowledge on how to design such hardware should help reduce the cost of neural network computing, and make sophisticated machine learning more accessible in the coming years.

As Moore's Law plateaus, one of the main paths forward is increasing customization for increased efficiency. This trend will paradoxically first arise at the edge (vs. in the data center), where hardware efficiency is most critical. Unfortunately, the need for customization/efficiency  runs  contrary  to  the  very  fast  evolution  of  machine-learning  algorithms.  Traditional  architectural approaches for achieving generality, while great for general-purpose computing, may not be best suited for resolving this tension between efficiency, velocity and generality.

Beyond hardware efficiency challenges, the other key challenge remains access to data. Consumer data privacy, corporate data confidentiality, or even regulatory compliance force a shift towards processing data closer to where it exists, i.e., at the edge. For many applications, doing so also provides useful, if not indispensable, latency, bandwidth and connectivity benefits for many applications. It even provides an out-of-the-box way to tackle the economic consequences of a plateauing Moore's Law for fast growing data-center machine-learning applications. The talk will go over these different trends and their consequences.